ABSTRACT

AI and deep learning applications are growing rapidly and need hardware that is fast, small, and uses less power. In AI systems, multiplication is the most commonly used math operation and takes up a lot of hardware resources. Approximate computing is a method where we allow the hardware to make very small errors in calculations. Since AI applications can still work correctly with tiny errors, approximate multipliers help reduce power, area, and hardware complexity.

This project designs and analyzes a 4-bit × 4-bit approximate multiplier for low-power AI use. First, an exact multiplier is designed in Verilog HDL using the normal partial-product method. This exact multiplier always gives the correct answer and is used as the reference for comparison. Then, an approximate multiplier is designed by removing some of the smaller partial products and keeping only the important larger ones. This makes the circuit simpler and more power-efficient.

Both multipliers are written in Verilog HDL and tested using Icarus Verilog. A testbench checks all possible inputs and GTKWave is used to view the simulation waveforms. Both designs are then synthesized on Xilinx Vivado to measure how many logic blocks (LUTs) are used, how fast the circuit runs, and how much power it consumes. Python is used to compare the outputs of both multipliers and calculate how much error the approximate multiplier introduces.

The main goal of this project is to study the trade-off between hardware savings and accuracy, and to show that approximate multipliers are a suitable and practical choice for low-power AI hardware applications.

Keywords: Approximate Computing, Approximate Multiplier, AI Applications, Partial Product Truncation, Verilog HDL, Low-Power VLSI.

LIST OF ABBREVIATIONS

| S. NO | ABBREVIATION | FULL FORM |
| --- | --- | --- |
| 1 | AI | Artificial Intelligence |
| 2 | DNN | Deep Neural Network |
| 3 | CNN | Convolutional Neural Network |
| 4 | MAC | Multiply-Accumulate (operation) |
| 5 | VLSI | Very Large Scale Integration |
| 6 | HDL | Hardware Description Language |
| 7 | RTL | Register Transfer Level |
| 8 | PPR | Partial-Product Reduction |
| 9 | FPGA | Field Programmable Gate Array |
| 10 | ASIC | Application Specific Integrated Circuit |
| 11 | LUT | Look-Up Table |
| 12 | VCD | Value Change Dump |
| 13 | ED | Error Distance |
| 14 | RED | Relative Error Distance |
| 15 | MED | Mean Error Distance |
| 16 | NMED | Normalized Mean Error Distance |
| 17 | ER | Error Rate |
| 18 | MRED | Mean Relative Error Distance |
| 19 | IoT | Internet of Things |
| 20 | DSP | Digital Signal Processing |
| 21 | PP | Partial Product |

LIST OF FIGURES

| FIGURE NO | NAME OF THE FIGURE | PAGE NO |
| --- | --- | --- |
| 4.1 | Exact Multiplier Block Diagram | 12 |
| 5.1 | Approximate Multiplier Block Diagram | 16 |
| 6.1 | Representative GTKWave Simulation Trace | 21 |
| 7.1 | Error Distance Frequency Distribution | 25 |
| 8.1 | Exact Multiplier – Vivado Synthesized Design View | 28 |
| 8.2 | Approximate Multiplier – Vivado Synthesized Design View | 28 |
| 8.3 | Vivado Post-Synthesis Resource Comparison | 29 |
| 9.1 | Exact vs Approximate Matrix Multiplication Results | 33 |

LIST OF TABLES

| TABLE NO | NAME OF THE TABLES | PAGE NO |
| --- | --- | --- |
| 2.1 | Standard Error Metrics in Approximate Computing | 6 |
| 3.1 | Software Tools Used in This Project | 9 |
| 4.1 | Worked Example – 10 × 12 = 120 | 13 |
| 5.1 | Partial Product Treatment – Exact vs Approximate | 16 |
| 5.2 | Worked Example Showing Zero Error for This Case | 17 |
| 7.1 | Error Metric Summary (N = 256 test vectors) | 24 |
| 8.1 | Post-Synthesis Utilization – Exact Multiplier | 27 |
| 8.2 | Post-Synthesis Utilization – Approximate Multiplier | 27 |
| 8.3 | Synthesis Comparison (Artix-7, xc7a35tcpg236-1) | 29 |
| 9.1 | 2×2 Matrix Multiplication – Exact vs Approximate | 32 |

TABLE OF CONTENTS

Abstract	I

List of Abbreviations	II

List of Figures	III

List of Tables	IV

Chapter 1: Introduction	2

1.1 Motivation	2

1.2 Problem Statement	2

1.3 Objectives	3

1.4 Scope of the Project	3

1.5 Organization of the Report	3

Chapter 2: Existing Work	5

2.1 Exact Multipliers	5

2.2 Approximate Computing: Concept and Motivation	5

2.3 Approximate 4:2 Compressor Designs in the Literature	5

2.4 Error Metrics in Approximate Computing	6

2.5 Approximate Multipliers in AI and Deep Neural Network Applications	6

2.6 Research Gap in Existing Literature	7

Chapter 3: System Requirements & Tools	9

3.1 Hardware Requirements	9

3.2 Software Requirements	9

3.3 Operating System Compatibility	9

3.4 Installation Commands (Ubuntu)	9

3.5 Verification Commands	10

Chapter 4: Exact Multiplier Design	12

4.1 Architecture Overview	12

4.2 Partial Product Generation	12

4.3 Verilog Implementation	13

4.4 Functional Description	13

4.5 Worked Example	13

4.6 Testbench Strategy	14

4.7 Verification Result	14

Chapter 5: Approximate Multiplier Design	16

5.1 Approximation Strategy	16

5.2 Verilog Implementation	17

5.3 Design Rationale	17

5.4 Worked Example (Same Inputs as Chapter 4)	17

5.5 Hardware Benefits	18

5.6 Design Trade-off Summary	18

Chapter 6: Simulation Results	20

6.1 Common Testbench Architecture	20

6.2 Stimulus Generation	20

6.3 Compilation and Execution	21

6.4 GTKWave Waveform Inspection	21

6.5 Console Output Summary	22

Chapter 7: Error Analysis	24

7.1 Python Analysis Pipeline	24

7.2 Error Metrics Computed	24

7.3 Graph: Error Distance Distribution	25

7.4 Discussion and Interpretation	25

Chapter 8: Synthesis Results	27

8.1 Vivado Synthesis Flow	27

8.2 Project Setup Steps	27

8.3 Utilization Report – Exact Multiplier	27

8.4 Utilization Report – Approximate Multiplier	27

8.5 Synthesized Design Views (Vivado)	28

8.6 Synthesis Comparison Chart	29

8.7 Comparison Table	29

8.8 Analysis of Results	30

8.9 Scalability Considerations	30

Chapter 9: AI Application Demo	32

9.1 Why Matrix Multiplication?	32

9.2 Demo Setup	32

9.3 Python Implementation	32

9.4 Results	32

9.5 Quality Impact Analysis	33

9.6 Relevance to Edge-AI Accelerator Design	33

Chapter 10: Conclusion & Future Work	35

10.1 Summary	35

10.2 Key Achievements	35

10.3 Conclusions Drawn	35

10.4 Limitations	36

10.5 Future Work	36

10.6 Tools Used — Summary	36

10.7 Closing Remark	37

References	38